AP Physics Chapter 12 -  Structures

In this unit,  4 basic notions are used to analyze the stress on nearly anything that is used to support a weight.  We often model our structures with truss systems but all systems will behave with the same basic principles.  if they are not truss systems, they can be more difficult but use the same approach.

A typical steel truss roof system

Most engineering majors will have to take a statics class.  The principles they use are the same as we use here.  As we have noted all year, a good grasp of some fundamental concepts allows you to solve some very challenging problems.

20 N

20 N

In all these concepts, Statics means the items is not moving and not beginning to move.  This requires that there be no net force on it.  So in this section, the forces must all balance.    As we have noted before there are two types of forces that we are concerned with:  1) Tension  and 2) Compression.  the key to working with these 2 forces properly is to remember that tension means pulling in on BOTH ends and compression is pushing out on BOTH ends.  We may draw these from the perspective of the item like this:  Tension   of 20 N . Tension pulls the same on both ends

or :  Compression   of 20 N .   Compression pushes the same on both ends

20 N

20 N

carbon steel can be made either BCC or FCC

The most prominent building materials  are concrete  (powdered dehydrated rock which binds with Calcium carbonate )  and Low carbon steel  ( Iron with a small amount carbon to fill interstial voids) .   They are remarkably well matched materials because the thermal expansion is the same. That is, they both expand and contract with temperature the same.   Concrete is usually great in compression and terrible in tension. Steel is great in both tension and compression but very heavy and very expensive.

A bridge wall rebar cage ready to be formed and poured

This is why we build so many things with concrete for compression and steel for tension.    A typical bridge  or highway wall has a construction build around a steel rebar (reinforcement bars)  cage.  This combination of tension and compression makes for great strength.  Without the rebar, the wall will have little strength.  With the rebar, the wall has amazing strength.

concrete cylinder samples for testing

Hydraulic  concrete stress test machine

Cement is a mixture of stones, sand and concrete  (powdered rock with lime  aka calcium carbonate) . Rocks and sand are way cheaper than the concrete . The two most common things that make concrete weak are 1) not adding enough cement because sand is cheaper and 2) adding too much water to the mixture.  Some things make concrete weaker such as adding too much water or not enough powdered rock.   In most large scale building in this country samples of the concrete are taken from the job site and crushed to prove they were adequate strength.

lateral shear pancaked slabs

crumbled concrete buildings in Haiti

We use the term "substandard" to describe what happens when houses are built without these considerations.  In Haiti, many homes were built without rebar and out of concrete that was very weak.   An earthquake will stress the structure, causing those without ability to bear either tension or compression to crumble.  Some were built with some rebar but without consideration for engineering design.  In these, lateral motion causes the building to "pancake"

Concept 1: Compressions push out on both ends  Tensions pull on both ends.

If a structure is static ( doesn't move )

Concept 2:  the sum of all Forces up =   Forces down

Concept 3: the sum of Forces Right =  Forces left

Concept 4:  the sum  of all Torque clockwise = torque counter clockwise.

Of these concepts, the toughest one to apply is the Torque concept. the Fancy language is "vector cross product" but let's just note the vectors twist things  so just to say T = F d is not enough.

Torque Force and distance must be at right angles .  In other words, it Torque is force x perpendicular distance to the pivot point.   = F x d.  The distance must be measured along the line of action

example 1: 					example 2:

Example 3:					example 4:

example 5:  					example 6:

Basic truss analysis :  Most trusses are drawn with "simple supports.  That is : they have a pin point on one side and a bearing joint on the other.  This allow for analysis if forces come in sideways.  Sideways forces we will leave for your engineering courses but we will use the method anyway.

Method of Joints

Method of joints looks at forces at single points in a structure.  If the point is not moving, it must be that the forces up = forces down and right = left.    We can conclude that members of the structure at right angles to force cannot bear any of the load.   For example in the triangle below, the point on the left   (called a node)  has a force pushing up.  It must be carried by  a component member A,  not member B cause it is perpendicular.

Truss analysis is often done with method of joints.  Use method of joints to find the force on each of the truss members loaded below.  Start by finding the forces on both ends with a rotation balance:

Start with the left joint 1 to find force  B and F:

Move over to Joint 2 on the right to find force E and G:

How could we find  forces A and C

Continue to force D and make sure they add up properly.

Buzenpuken

Bar and

Grill

Many times Method of joints makes a very quick and easy problem: Consider a 40 Kg sign hanging from a wall from a solid bar and a wire under tension.  The wire makes an angle of 45 degrees.

In a second sample.  Consider how you could find forces on a ladder tilted at 60 degrees  if the ladders mass if 25 Kg and the 90 Kg person on the ladder is 4 ft from the bottom.

One of your homework problems (level III) uses a non- traditional approach to loading a truss:

A solution to the problem is best done like this:

Method of Sections:

In larger truss systems, method of sections may bypass a lot of crazy work.  It can also be used for smaller systems but the benefit isn't as obvious.  Consider this example which is hard to do with method of joints:

Steps for method of sections:

- find the reactive forces from the supports.

- Imagine a slice through the truss.

- Pick a pivot point which is "convenient" for the member you are trying to analyze.

- use Torques to find why the section does not rotate around the chosen pivot point.

Q

P

N

L

K

J

I

G

H

F

E

D

C

B

A

Example - Method of sections

O

M

U

Zero force members.

A

F

C

E

D

B

G

Mechanical Buckling:

Examples of zero force members:

Kansas City Hyatt hotel atrium bridge collapse